National Textile University

Department of Computer Science

Project:

Fingerprint Attendance System

Submitted to:

Sir Nasir Mehmood

Submitted by:

Arooj Komal (23-NTU-CS-1140)

Ayesha Khalid (23-NTU-CS-1248)

Faseeha Ahmad (23-NTU-CS-1251)

Subject:

Embedded IoT Systems

Section:

BSAI

Semester:

5th

Contents

IoT Based Fingerprint Attendance System	3

Using ESP32, AS608 Fingerprint Sensor, Blynk, Node-RED and Google Sheets	3

1. Executive Summary	3

2. Introduction	3

3. Project Objectives	3

4. System Overview	3

4.1 System Architecture	3

4.2 Hardware Components	4

4.3 Software Components	4

4.4 Blynk Widget Configuration	4

4.5 Working Principle	4

4.6 Data Flow	5

4.7 Hardware Pin Connections	5

4.8 Circuit diagram	6

4.8 Advantages	6

4.9 Applications	6

5. Phase 1: Implementation Using Blynk	7

Libraries used:	7

Features Implemented:	7

Limitations of Blynk:	7

6. Phase 2: Implementation Using Node-RED	7

Advantages of Node-RED:	7

Node-RED Functions:	7

7. System Working Principle	8

8. Google Sheets Data Structure	8

9.Flowchart	9

10.Screenshots	10

11.Results and Observations	12

12. Testing and Validation	13

13. Conclusion	13

14. Future Enhancements	13

# IoT Based Fingerprint Attendance System

### Using ESP32, AS608 Fingerprint Sensor, Blynk, Node-RED and Google Sheets

## 1. Executive Summary

This project presents an IoT-based Fingerprint Attendance System that was implemented in two phases. In the first phase, the system was developed using the Blynk IoT platform for dashboard control and monitoring. In the second phase, the system was enhanced and migrated to Node-RED, providing greater flexibility, control, and customization.

The system uses an ESP32 microcontroller, AS608 fingerprint sensor, and OLED display to authenticate users biometrically and automatically record attendance. Attendance data is stored in Google Sheets with real-time date and time fetched via the internet (NTP). The final solution is secure, automated, and suitable for educational institutions and organizations.

## 2. Introduction

Traditional attendance systems are manual, time-consuming, and prone to errors such as proxy attendance. Biometric authentication provides a reliable solution by uniquely identifying each individual.

This project aims to design and implement an IoT-enabled fingerprint attendance system that records attendance automatically and stores it in the cloud. The project was first implemented using Blynk and later improved using Node-RED to overcome limitations and add advanced control.

## 3. Project Objectives

- To design a fingerprint-based attendance system

- To implement automatic enrollment of new users

- To authenticate users using biometric fingerprints

- To provide real-time feedback using OLED display

- To store attendance records automatically in Google Sheets

- To develop IoT dashboards using Blynk and Node-RED

- To prevent duplicate attendance entries on the same day

## 4. System Overview

### 4.1 System Architecture

The system consists of four main layers:

- Hardware Layer – ESP32, AS608, OLED

- Communication Layer – WiFi

- Cloud Layer – Blynk & Google Sheets

- User Interface Layer – Blynk Dashboard

### 4.2 Hardware Components

#### ESP32

Controls the system, manages WiFi, processes fingerprints, and sends data to the cloud.

#### AS608 Fingerprint Sensor

Used for fingerprint enrollment and authentication. Each fingerprint is stored with a unique ID.

#### OLED Display

Shows system status such as booting, WiFi connection, enrollment instructions, and attendance confirmation.

### 4.3 Software Components

- Blynk Platform: Dashboard control, status display, and user input

- Google Sheets: Cloud database for attendance records

- Arduino Framework: Used to program ESP32

### 4.4 Blynk Widget Configuration

| Widget | Virtual Pin | Purpose |
| --- | --- | --- |
| Enroll Button | V0 | Start enrollment |
| Attendance Button | V1 | Mark attendance |
| Status Label | V2 | Show system status |
| Name Label | V3 | Display user name |
| Name Input | V4 | Enter name for enrollment |

### 4.5 Working Principle

- System powers ON

- ESP32 connects to WiFi

- User enters name in Blynk

- User presses Enroll

- Fingerprint is scanned and stored

- User presses Attendance

- Fingerprint is verified

- Attendance is uploaded to Google Sheets

- OLED shows confirmation

### 4.6 Data Flow

Fingerprint → ESP32 → WiFi → Blynk Cloud → Google Sheets → Dashboard

### 4.7 Hardware Pin Connections

#### AS608 Fingerprint Sensor → ESP32

| AS608 Pin | ESP32 Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TX | GPIO 16 |
| RX | GPIO 17 |

#### OLED Display (I2C) → ESP32

| OLED Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

#### Power Summary

- ESP32: USB 5V

- AS608: 5V

- OLED: 3.3V

### 4.8 Circuit diagram

### 4.8 Advantages

- Automatic attendance

- Real-time monitoring

- Cloud storage

- Easy to use dashboard

- Reduced manual errors

### 4.9 Applications

- Schools

- Colleges

- Offices

- Training centers

## 5. Phase 1: Implementation Using Blynk

In the first phase, the project was implemented using the Blynk IoT platform.

### Libraries used:

- adafruit/Adafruit Fingerprint Sensor Library

- adafruit/Adafruit SSD1306

- adafruit/Adafruit GFX Library

- blynkkk/Blynk

### Features Implemented:

- Enroll button to register new users

- Text input widget for user name

- Attendance button to mark attendance

- OLED feedback messages

- Google Sheets integration

### Limitations of Blynk:

- Limited widget customization

- Dependency on mobile application

- Less flexibility in logic handling

- Difficulty in handling complex workflows

Due to these limitations, the project was further enhanced using Node-RED.

## 6. Phase 2: Implementation Using Node-RED

In the second phase, Node-RED was used to replace Blynk as the dashboard and control interface.

### Advantages of Node-RED:

- Fully customizable web dashboard

- Better control over logic and data flow

- Easier integration with Google Sheets

- Real-time visualization and debugging

### Node-RED Functions:

- Trigger enrollment and attendance

- Display system status

- Prevent duplicate attendance

- Send data to Google Sheets

## 7. System Working Principle

- ESP32 connects to Wi-Fi

- Fingerprint sensor and OLED are initialized

- User enrolls fingerprint (two scans)

- Fingerprint ID is mapped to user name

- User scans finger for attendance

- ESP32 verifies fingerprint

- Date and time fetched using NTP

- Attendance sent to Google Sheets

- Duplicate attendance is prevented

## 8. Google Sheets Data Structure

| Finger_ID | Name | Date | Time | Status |
| --- | --- | --- | --- | --- |
| 1 | Ali | 07-01-2026 | 09:15 | Present |

Google Apps Script updates existing records if attendance is already marked for the day.

## 9.Flowchart

## 10.Screenshots

## 11.Results and Observations

- Accurate fingerprint authentication

- Real-time attendance logging

- Correct date and time from NTP server

- No duplicate attendance entries

- Smooth dashboard operation in Node-RED

- OLED provides clear user feedback

## 12. Testing and Validation

| Test Case | Result |
| --- | --- |
| Valid fingerprint | Attendance marked |
| Same user twice | Duplicate prevented |
| Invalid fingerprint | Access denied |
| Wi-Fi disconnection | Auto reconnection |
| Power reset | System resumes |

## 13. Conclusion

The IoT-based Fingerprint Attendance System successfully automates attendance management using biometric authentication and cloud storage. Implementing the project in two phases—first with Blynk and then with Node-RED—helped overcome platform limitations and improved system reliability and flexibility.

## 14. Future Enhancements

- Face recognition integration

- Offline attendance storage

- SMS / Email notifications

- Admin web portal

- Mobile application